SCOPE OF WORKS

| File ID: | F-M MoE JLS Rehab works |
| --- | --- |
| SUB PROJECT NAME: | Rehabilitation Of Works At The Ministry Of Education. |
| LOCATION: | Kismayo District, Jubaland State |
| DESCRIPTION OF PROJECT: | The Proposed Project Will Involve The Rehabilitation Of Existing Offices, Partition Of Existing Hall To Office And Store, Rehabilitation Of Existing Canteen, Rehabilitation Of Existing Mosque, And Construction Of Two Latrine Blocks Each With Two Units. |

GENERAL

Concern Worldwide under the GPE-ESPIG program plans to rehabilitate existing facilities in the ministry of Education premises in Banadir and Jubaland including Teachers training institute. This is part of the plan aimed at improving the access, quality and well being of the users at the MoE.

Concern Worldwide has been tasked to undertake activities related to the programme in two regions namely Banadir and Jubaland. In Jubaland, rehabilitation of the Ministry of Education premise and Jubaland Teachers Training Institute is planned.

SCOPES OF WORK

Rehabilitation works shall cover four main buildings. Block A, Block B, Canteen block with ablution, Mosque and Construction of two new latrine blocks each with two units.

The Scope of Works to this Subproject shall include but not limited to the following activities:

All the works shall commence with Site Mobilization, Preparation, Clearing and provision of all materials, labor and equipment to the project site.  The specific scope of works shall be as follows.

- Block A: Rehabilitation of Existing Office block to create Verandah & Roofing and Gutter drain system.

- Excavate striped foundation in normal soils average depth of 700mm to a total length of 43meters. And width of 600mm to cast foundation strip

- Excavate 17No column base each 600x600 mm

- Blinding for all excavated strip foundation and column base with 50mm concrete ratio 1:4:8 in parts of cement sand and aggregate.

- Vibrate reinforce concrete to the strip foundation 600mm and 200mm thick at the trench and 600x 600mm and 200mm thick at 17 no. column  footing.

- Construct sub structure wall and sub structure column

- Cast 17 columns and construct parapet wall

- Cast floor for the extended verandah

- Roof the extended verandah, install ceiling and decorative lighting

- Construct access ramp to the verandah

- Plaster and paint works

- Repair of exiting gutters and downpipes.

- BLOCK B-Partition into Office and Store and construction of new verandah

- Remove existing roof and truss, remove the doors and windows, hack the floor

- Demolish existing floor in a strip of 600mm wide to create a foundation trench, this will divide the room into two.

- Excavate striped foundation in normal soils average depth of 700mm to a total length of 12meters. And width of 600mm to cast foundation strip

- Excavate 10No column base each 600x600 mm

- Blinding for all excavated strip foundation and column base with 50mm concrete ratio 1:4:8 in parts of cement sand and aggregate.

- Vibrate reinforce concrete to the strip foundation 600mm and 200mm thick at the trench and 600x 600mm and 200mm thick at 10 no. column  footing.

- Construct sub structure wall and sub structure column, and cast the floor

- Cast 10No columns 1No. square section and 9No. circular section)and construct partition  wall and the parapet wall

- Add grade beam for partition wall, and lintol for entire wall (partition wall and external walls)

- Roof the building including all the timber works and ceiling

- Construct a ramp access to the building

- Supply and install all the windows and doors

- Electrical works installing lighting points, sockets and switches in the store and office

- Plaster and paint works

- Repair of exiting gutters and downpipes.

- Supply materials and assemble shelves at the store.

- Canteen Block With Ablution

- Site Mobilization and Clearance incluidng removal damaged material,floors, iron sheets  Doors and windows

- Hack the existing floor to remove existing old cement floor.

- Cast new floor 150mm thick with concrete mix 1:2:4 in parts of cement sand and aggregate.

- Hack the existing wall with crack and plaster

- Supply and cast lintols and ring beam

- Supply and install doors and windows

- Supply and fix  28 IT5 Gauge Galvanized corrugated  Iron sheet pre-painted . Secured with approved accessories GCI sheet including timber trusses

- Supply and fix interior decoration ceiling design, 6 mm thick plywood laminated ceiling

- Painting and general finishes

At the ablution block:

- Remove badly damaged doors and windows and replace with new ones

- Plater all the walls internally and externally.

- Supply and fix all the piping works and sanitary installation.

- Provide for internal lights and lumps

- Block C: Mosque Block

- This block requires extension of the existing building. The specific tasks are:

- Wall demolition for the existing wall to allow for extension

- Excavate a striped foundation to a depth of 700mm and construct sub structure and super structure extension wall.

- Supply materials and cast columns grade beam floor slab.

- Supply and fix 28 IT5 Gauge Galvanized corrugated Iron sheet pre-painted. Secured with approved accessories GCI sheet including timber trusses

- Supply and fix interior decoration ceiling design, 6 mm thick plywood laminated ceiling

- Painting and general finishes

- At the ablution area, allow for pipe works.

- Hack existing floor and install ceramic tiles.

- Construction Of Two New Latrines Block (2 Units)

- For the two blocks of latrine, construct according to the specifications in the

PERIOD OF PERFORMANCE

All Works under this Subproject shall be completed in period of two (2) month reckoned one week after the date of issuance of the Notice to Proceed. The Contractor shall expeditiously work to complete the Work requirements in accordance with the Drawings, Specifications, Terms and Conditions of the Subcontract.

APPLICABLE CODES AND STANDARDS

The Standards being required by the Specifications and Contract Documents is that  all  work and materials  shall meet the  requirement  of  the Uniform Building Code of the United States, and/or applicable American Society for Testing Materials  (ASTM),  American Association of State Highway and Transportation Officials (AASHTO) Specifications, the latest  American Concrete Institute  Manual of Concrete Practice, American Institute of Steel Construction (AISC) Manual for relevant to the Works except in cases where the Agreed Building Code requires a higher standard.  In such cases BS Code shall govern.  Where the abbreviations listed below are used, it refers to the latest code, standards, or publication of the following organizations:

AASHTO	American Association of State Highway and Transportation Officials

ACI		American Concrete Institute

AISC		American Institute of Steel Construction

ANSI		American National Standards Institute

ASA		American Standard Association

ASCE		American Society of Civil Engineers

ASTM		American Society for Testing and Material

AWS		American Welding Society

BSI		British Standards Institute

ICAO		International Civil Aviation Organization

BSICP		British Standard Institute Code of Practice

FAA		Federal Aviation Administration

PCA		Portland Cement Association

UBC		Uniform Building Code

Should the Contractor, at any time and for any specific reasons, wish to deviate from the above standards or desires to use materials or equipment other than those provided for by the above standards, then he shall state the exact nature of the change giving the reasons for making the change and shall submit complete specifications of the materials and descriptions of the equipment for the Concern Engineering Manager's approval, whose decisions shall be conclusive and binding upon the Contractor.